| Wilmort Park  Middletown Township (N.J.)  ADA Transition Plan Washington Ave. & Monroe St. Spring 2008 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location | Identified Barrier(s) | Recommended Corrective Action Based on ADAAG or Applicable Guideline | Priority: L=Low M=Medium H=High | *Cost Estimate | Barrier Removal Target Date & Notes |
| EXTERIOR FEATURES |  |  |  |  |  |
| Parking | One space is signed as accessible and one is striped as accessible. None are designated as “van accessible”. | At least 1 accessible space shall be designated as “van accessible” and be served with an access aisle at least 96” wide. Signs shall be located so they cannot be obscured by a vehicle parked in the space. Two accessible spaces may share a common access aisle (4.1.2(5)(b) & 4.6). | H | $500 |  |
| Accessible Paths of Travel | The route to the ball field, playground and basketball court is rough and uneven grass. | At least one accessible route shall connect accessible buildings, facilities, elements, and spaces that are on the same site. Ground and floor surfaces along accessible routes shall be stable, firm, slip-resistant and free of abrupt changes in level (4.3.2). | M+ | $7,500 |  |
| Play Ground | A threshold up to 5” encompasses the play equipment. The play equipment surface is engineered wood chips. | Changes in level up to ¼” may be vertical and without edge treatment. Changes in level between ¼” and ½” shall be beveled with a slope no greater than 1:2. Changes in level greater than ½” shall be accomplished by means of a ramp (4.5.2). Recommendation: this material has been generally accepted as “ADA compliant”, however, to maintain an adequate firm and level surface free of thresholds, this surface must be constantly maintained. We recommend using a rubber or corked surface. There are no ADA enforceable standards for playground areas, only guidelines at the present time; however, program access must still be met. | L+ L+ | $500 $3,000 |  |
| Ball Field | No designated wheelchair seating exists at the bleacher seating. | Recommendation: designate at least one location as wheelchair accessible seating. Ensure that the location provides an unobstructed line of sight. The designated wheelchair seating area should be large enough for at least 1 wheelchair user and 1 companion. A 60” by 60” minimum area would be preferred. Designation could be accomplished by painting the universal symbol of access on the pavement. | M | $1,000 |  |

| Priority Level | Cost Estimate Totals |
| --- | --- |
| Low’s | $3,500 |
| Medium’s | $8,500 |
| High’s | $500 |
| TOTAL | $12,500.00* |

* Cost estimates are given as general guidance only.  Actual construction costs will vary depending on availability of materials and contractors used and the type of barrier removal utilized.